Software Requirements Specification

AI-First Operations Platform

Textile Wholesale & Manufacturing — Surat

Prepared in accordance with IEEE Std 830 structure

Document version: 1.0 (Draft for sign-off)

Date: June 2026

Status: For client & engineering review

Related documents: High-Level Design v1; Technical Design & Costing v1

Table of Contents

1. Introduction	3

2. Overall Description	4

3. External Interface Requirements	6

4. Functional Requirements	7

4.1 Stock Inventory — Incoming & Outgoing	7

4.2 Job Cards & Folding Shortage	7

4.3 AI Capture & Confirmation	7

4.4 The Brain — Reports & Chat	8

4.5 Worker Efficiency & CCTV Monitoring	8

4.6 Access Control & Administration	9

5. Non-Functional Requirements	10

6. Data Requirements	11

7. Traceability & Acceptance	12

# 1. Introduction

## 1.1 Purpose

This Software Requirements Specification (SRS) defines the functional and non-functional requirements for an AI-first operations platform for a textile wholesaler and manufacturer in Surat. It is intended for the client (owner/stakeholders), the engineering team building the system, and any future maintainers. It is the authoritative reference against which the system will be designed, built, and accepted.

## 1.2 Scope

The system replaces manual data entry with AI-based capture and provides automated reporting, a conversational query interface, and camera-based worker-efficiency monitoring. It covers three operational problems: stock inventory (incoming/outgoing), job-card meter updates with folding-shortage calculation, and worker efficiency (allotted vs done, with flagging).

In scope: photo-based capture of meters/lots/challans; a Postgres system of record; an AI brain for daily reports and natural-language queries over firm history; CCTV-based activity monitoring fused with job-card output; web and mobile (PWA) interfaces with role-based access.

Out of scope (v1): accounting/billing/GST software replacement; direct machine/PLC control; automated payroll actions; e-commerce or customer-facing portals.

## 1.3 Definitions, Acronyms, Abbreviations

| Term | Meaning |
| --- | --- |
| Lot | A batch of fabric tracked as a unit through the firm |
| Challan | Delivery/dispatch note accompanying goods movement |
| Job card | Record of a production stage: meters in, meters out, worker, shortage |
| Folding shortage | Meters_in minus meters_out at the folding stage |
| Capture event | A single photo submitted for AI reading |
| The Brain | AI agent providing auto-reports and chat over the database |
| Confidence gate | Threshold below which an AI reading needs human confirm |
| PWA | Progressive Web App (installable, mobile-friendly web app) |
| Edge box | On-site GPU device running CCTV analytics locally |
| RBAC | Role-Based Access Control |

## 1.4 References

- IEEE Std 830-1998, Recommended Practice for Software Requirements Specifications.

- Project High-Level Design document, v1 (June 2026).

- Project Technical Design & Costing document, v1 (June 2026).

## 1.5 Overview

Section 2 gives the overall product perspective, users, and constraints. Section 3 specifies external interfaces. Section 4 details functional requirements by module. Section 5 covers non-functional requirements. Section 6 lists data requirements. Section 7 provides the requirements traceability matrix and acceptance criteria.

# 2. Overall Description

## 2.1 Product Perspective

The platform is a new, self-contained system. Three input channels (worker photo capture, CCTV streams, and admin chat) feed a single backend and Postgres database that acts as the system of record. AI services for vision, the reasoning brain, and video analytics run as independent modules behind the backend. A web/PWA presentation layer serves owner, supervisors, and workers according to role.

## 2.2 Product Functions (summary)

- Capture incoming and outgoing stock by photographing challans/lot tags; AI extracts lot, quality, and meters.

- Capture job-card folding meters; AI records meters-out and computes shortage automatically.

- Hold low-confidence readings for one-tap human confirmation before committing.

- Generate an automated end-of-day report without manual assembly.

- Answer natural-language questions over the full firm history via a chat assistant.

- Monitor worker activity via CCTV, fuse it with job-card output, and flag low efficiency.

- Enforce role-based access for owner, supervisor, and worker.

## 2.3 User Classes and Characteristics

| User class | Characteristics | Primary activities |
| --- | --- | --- |
| Owner / Admin | Low technical skill; decision-maker | View dashboards & reports; chat queries; review flags |
| Supervisor | Moderate; floor-level | Confirm low-confidence reads; manage allotments; view section |
| Worker / Operator | Low; high volume of use | Capture photos at stock/job-card stages |
| System Admin (IT) | Technical | User management, configuration, maintenance |

## 2.4 Operating Environment

- Capture clients: modern mobile/tablet browsers (PWA), Android-first.

- Admin/supervisor: desktop and tablet browsers.

- Backend: Linux container host; Node.js/TypeScript; Postgres (Supabase).

- CCTV analytics: on-site edge GPU device (recommended) on the factory LAN.

- Connectivity: intermittent internet tolerated; capture queues locally and syncs.

## 2.5 Design and Implementation Constraints

- Database is the single source of truth; AI must never assert figures not backed by a query.

- Chat queries run under a read-only database role; no AI-initiated writes.

- Vision provider must be abstracted so a hybrid OCR path can be added without rework.

- Worker video should remain on-premises where feasible (privacy/labour considerations).

- UI text and capture flows must accommodate English, Hindi, and Gujarati numerals.

## 2.6 Assumptions and Dependencies

- Scale: ~18 cameras and ~300–500 capture events/day (medium).

- Cloud multimodal LLM (Claude) availability for vision and the brain.

- Adequate camera placement and lighting at each workstation.

- Client agrees a worker-monitoring policy (coaching vs punitive) before go-live.

# 3. External Interface Requirements

## 3.1 User Interfaces

- Worker capture (PWA): camera-first screen, large capture button, instant preview of AI reading, one-tap confirm/correct, offline queue indicator.

- Supervisor console: pending-confirmation queue, allotment management, section dashboards, flags.

- Owner dashboard: live stock/shortage/efficiency tiles, daily report viewer, chat panel.

- Accessibility/locale: multilingual labels; high-contrast, touch-friendly controls for floor use.

## 3.2 Hardware Interfaces

- Mobile/tablet cameras for capture.

- IP/CCTV cameras (RTSP) feeding the edge analytics device.

- Edge GPU device interfacing with the backend over the LAN/API.

## 3.3 Software Interfaces

| Interface | Type | Purpose |
| --- | --- | --- |
| Claude API | HTTPS / REST | Vision extraction, report writing, chat reasoning |
| Supabase/Postgres | SQL / client lib | System of record, auth, storage, pgvector |
| Object storage | S3-compatible | Photos and short CCTV clips |
| Message/queue | Redis | Capture jobs, async processing |
| Notification | WhatsApp/email API | Daily report and flag delivery (optional) |

## 3.4 Communications Interfaces

- HTTPS/TLS for all client–backend and backend–API traffic.

- WebSocket for live dashboard updates and chat streaming.

- RTSP within the LAN for camera-to-edge streaming; no raw video leaves the premises in the edge configuration.

# 4. Functional Requirements

Each requirement has a unique ID (FR-x.y) and a priority: M (must-have, v1), S (should-have), C (could-have/later). Priorities feed the phased delivery plan.

## 4.1 Stock Inventory — Incoming & Outgoing

| ID | Requirement | Pri | Phase |
| --- | --- | --- | --- |
| FR-1.1 | The worker shall capture incoming stock by photographing a challan or lot tag. | M | 1 |
| FR-1.2 | The system shall extract lot number, quality/design, and meters received from the photo. | M | 1 |
| FR-1.3 | The worker shall capture outgoing stock with dispatch meters, party, and lot. | M | 1 |
| FR-1.4 | The system shall record each movement as an IN/OUT ledger entry linked to its source photo. | M | 1 |
| FR-1.5 | The system shall maintain a running stock balance per lot and per quality, derived from the ledger. | M | 1 |
| FR-1.6 | The system shall prevent an outgoing entry that exceeds available balance, or flag it for review. | S | 1 |

## 4.2 Job Cards & Folding Shortage

| ID | Requirement | Pri | Phase |
| --- | --- | --- | --- |
| FR-2.1 | A supervisor shall create a job card linking a lot, process stage, and assigned worker, with meters-in. | M | 1 |
| FR-2.2 | The worker shall capture folded meters-out by photograph at the folding stage. | M | 1 |
| FR-2.3 | The system shall compute folding shortage as meters_in − meters_out automatically. | M | 1 |
| FR-2.4 | The system shall flag a job card whose shortage exceeds a configurable threshold. | S | 1 |
| FR-2.5 | The system shall track job-card status (open / in-process / folded / closed). | M | 1 |

## 4.3 AI Capture & Confirmation

| ID | Requirement | Pri | Phase |
| --- | --- | --- | --- |
| FR-3.1 | The system shall attach a confidence score to every AI reading. | M | 1 |
| FR-3.2 | Readings at or above the confidence threshold shall commit automatically to the ledger/job card. | M | 1 |
| FR-3.3 | Readings below the threshold shall be held in a pending queue for one-tap confirm or correction. | M | 1 |
| FR-3.4 | The system shall pre-filter blurry/irrelevant photos before sending them for full extraction. | S | 1 |
| FR-3.5 | The system shall retain the original photo, the extracted data, and the confirm action for audit. | M | 1 |
| FR-3.6 | The capture client shall queue submissions offline and sync when connectivity returns. | S | 1 |
| FR-3.7 | The vision provider shall be replaceable (e.g., hybrid OCR) via configuration without code rework. | S | 2 |

## 4.4 The Brain — Reports & Chat

| ID | Requirement | Pri | Phase |
| --- | --- | --- | --- |
| FR-4.1 | The system shall generate an end-of-day report (meters in/out, shortages, stock balance, efficiency) automatically. | M | 1 |
| FR-4.2 | The report shall be viewable on the dashboard and optionally pushed via WhatsApp/email. | S | 1 |
| FR-4.3 | The owner shall ask questions in natural language (English/Hindi/Gujarati) via chat. | M | 2 |
| FR-4.4 | The brain shall answer using read-only queries against the database, never fabricated figures. | M | 2 |
| FR-4.5 | The brain shall display the underlying numbers/records supporting each answer. | S | 2 |
| FR-4.6 | The system shall log every question, the query executed, and the answer for audit. | M | 2 |
| FR-4.7 | The brain shall answer historical/contextual questions using retrieval over past records. | C | 2 |

## 4.5 Worker Efficiency & CCTV Monitoring

| ID | Requirement | Pri | Phase |
| --- | --- | --- | --- |
| FR-5.1 | The system shall record work allotted per worker per shift/day. | M | 1 |
| FR-5.2 | The system shall compute efficiency as work done (meters) vs allotted, per worker per day. | M | 1 |
| FR-5.3 | The system shall flag workers below a configurable efficiency threshold. | M | 1 |
| FR-5.4 | CCTV analytics shall estimate active vs idle time per worker per station. | S | 3 |
| FR-5.5 | The system shall fuse camera activity with job-card output and flag only when both indicate low performance. | M | 3 |
| FR-5.6 | Flags shall be presented as coaching/planning signals, not automated pay actions. | M | 3 |
| FR-5.7 | CCTV processing shall run on-site (edge) in the recommended configuration. | S | 3 |

## 4.6 Access Control & Administration

| ID | Requirement | Pri | Phase |
| --- | --- | --- | --- |
| FR-6.1 | The system shall enforce RBAC: owner (all), supervisor (section), worker (capture only). | M | 1 |
| FR-6.2 | An IT admin shall manage users, roles, and configurable thresholds. | M | 1 |
| FR-6.3 | The system shall record an audit trail of edits, confirms, and admin actions. | M | 1 |

# 5. Non-Functional Requirements

## 5.1 Performance

| ID | Requirement | Target |
| --- | --- | --- |
| NFR-P1 | AI reading returned after capture (high-confidence path) | ≤ 5 s typical |
| NFR-P2 | Dashboard query / tile load | ≤ 2 s |
| NFR-P3 | Chat answer latency | ≤ 8 s typical |
| NFR-P4 | Sustained capture throughput | ≥ 500/day, burst 60/hr |
| NFR-P5 | CCTV analytics processing | near real-time, 18 streams |

## 5.2 Reliability & Availability

- NFR-R1: Application availability target 99% during working hours.

- NFR-R2: No capture data loss; offline submissions queued and synced (supports FR-3.6).

- NFR-R3: Edge CCTV continues functioning during internet outages.

- NFR-R4: Automated daily backups of the database with point-in-time recovery.

## 5.3 Security & Privacy

- NFR-S1: TLS for all traffic; encryption at rest for database and stored media.

- NFR-S2: RBAC enforced server-side; least-privilege roles (supports FR-6.1).

- NFR-S3: Chat/brain uses a read-only DB role; no AI-initiated writes (supports FR-4.4).

- NFR-S4: Worker video retained on-premises where feasible; configurable retention period.

- NFR-S5: Full audit logging of confirms, edits, admin actions, and chat queries.

## 5.4 Usability

- NFR-U1: A worker can complete a capture in ≤ 3 taps.

- NFR-U2: Multilingual UI (English/Hindi/Gujarati); icon-led for low-literacy users.

- NFR-U3: Touch-friendly, high-contrast UI suitable for factory-floor conditions.

## 5.5 Maintainability & Portability

- NFR-M1: AI services modular and independently deployable.

- NFR-M2: Vision provider abstracted behind one interface (supports FR-3.7).

- NFR-M3: PWA runs on any modern browser; no app-store dependency.

## 5.6 Scalability

- NFR-Sc1: Capture volume scalable to ~2× (≈800/day) without architectural change.

- NFR-Sc2: Additional cameras addable by adding/upgrading edge capacity.

# 6. Data Requirements

## 6.1 Core Entities

The database is the single source of truth. Stock, shortage, and efficiency are all derived from one movement ledger, so the figures can never disagree.

| Entity | Key attributes | Supports |
| --- | --- | --- |
| lots | lot_id, quality, design, grade, status | FR-1.x, FR-2.x |
| stock_movements | id, lot_id, direction, meters, party, source_doc, ts | FR-1.4, FR-1.5 |
| job_cards | id, lot_id, process, worker_id, meters_in, meters_out, shortage, status | FR-2.x |
| workers | id, name, section, role | FR-5.x |
| allotments | id, worker_id, job_card_id, meters_allotted, shift, date | FR-5.1, FR-5.2 |
| capture_events | id, photo_url, type, ai_json, confidence, status, confirmed_by | FR-3.x |
| efficiency_daily | worker_id, date, allotted, done, efficiency_pct, flagged | FR-5.2, FR-5.3 |
| cctv_activity | worker_id, station, active_pct, idle_min, ts | FR-5.4, FR-5.5 |
| chat_audit | id, user_id, question, sql_run, answer, ts | FR-4.6 |
| users | id, name, role, locale, active | FR-6.x |

## 6.2 Retention & Integrity

- Ledger and job-card records are append-only with audited corrections (no silent overwrite).

- Photos retained for a configurable period; CCTV clips lifecycle-expired to control storage.

- Referential integrity enforced between lots, movements, job cards, and allotments.

# 7. Traceability & Acceptance

## 7.1 Problem-to-Requirement Traceability

| Client problem | Requirements | Phase |
| --- | --- | --- |
| Stock inventory (in/out) | FR-1.1 … FR-1.6, FR-3.x | 1 |
| Job card meter update + shortage | FR-2.1 … FR-2.5, FR-3.x | 1 |
| Worker efficiency monitor + flag | FR-5.1 … FR-5.7 | 1 & 3 |
| AI brain (reports + chat) | FR-4.1 … FR-4.7 | 1 & 2 |
| Access & audit | FR-6.1 … FR-6.3 | 1 |

## 7.2 Acceptance Criteria (representative)

- AC-1: Given a clear printed challan, capture extracts lot, quality, and meters correctly and commits without manual typing (FR-1.1–1.2).

- AC-2: Given a low-confidence reading, the system holds it for confirm rather than committing wrong data (FR-3.1–3.3).

- AC-3: At folding, shortage equals meters_in minus meters_out with no manual calculation (FR-2.3).

- AC-4: At day end, a report is produced automatically with no human assembly (FR-4.1).

- AC-5: A plain-language question returns an answer backed by a real query and shows the numbers (FR-4.3–4.5).

- AC-6: A worker is flagged only when both low activity and low output coincide (FR-5.5).

- AC-7: A worker cannot access another section’s data; a supervisor cannot perform admin actions (FR-6.1).

## 7.3 Open Items to Confirm Before Build

- Confidence threshold value(s) and who clears the pending queue.

- Efficiency and shortage flag thresholds.

- Edge vs cloud for CCTV (edge recommended) and camera count.

- Worker-monitoring policy and data-retention periods.

- Report delivery channel (dashboard only vs WhatsApp/email).

End of Software Requirements Specification v1.0.